TOMORROW SESSION RUNBOOK

# Unseen Subject 10 Live Model Tests

Five leakage-guarded decoder runs with automatic diagnostic recording

Run every command from the project root in PowerShell. Keep Subject 10 out of training by using the supplied checkpoints and the --require-unseen guard. Use the same maze and similar block duration for every model.

## Before the participant arrives

[ ]  Open PowerShell in C:\Users\mourad\OneDrive\Documents\eegthingy-new

[ ]  Synchronize the environment if needed:  uv sync --frozen

[ ]  Close OpenBCI GUI and any program that may hold the Cyton COM port.

[ ]  Charge the board, confirm PC mode, attach Daisy, and prepare electrode gel.

## Real cap preflight

uv run python scripts/mirepnet_live_preflight.py --subject 10 --hardware --checkpoint models/mirepnet/mirepnet__7subjects_28runs_22b10fee.pt

[ ]  Do not start testing until all connected channels pass or every flagged channel has been inspected.

[ ]  Write down the selected COM port: __________________________

[ ]  Use standard maze number: ______    View: ______    Model order: __________________

## Rules for a fair comparison

[ ]  Use the exact checkpoint printed on each model page.

[ ]  Use the same cap placement, maze, instructions, and approximate duration.

[ ]  Give the participant a short rest between models; note any cap adjustment.

[ ]  Never select an S10 recording as training data. The guard will reject leaked checkpoints.

| Item | Value |
| --- | --- |
| Labels | 1 = left hand imagery; 2 = right hand imagery |
| Live log location | recordings/diagnostic-sessions/ |
| Recommended baseline | EA-FBCSP |
| Foundation experiment | MIRepNet |

SUBJECT 10 UNSEEN MODEL TEST

# EA-FBCSP

Fast classical baseline  |  82.02% strict unseen balanced accuracy

## Run the maze

uv run python classifier/run.py --live-maze --decoder ea --subject 10 --require-unseen --checkpoint models/classical/ea__7subjects_28runs_22b10fee.joblib

| Item | Value |
| --- | --- |
| Checkpoint | ea__7subjects_28runs_22b10fee.joblib |
| Training subjects | 1, 3, 4, 5, 6, 7, 8 only |
| Startup calibration | 45 seconds; covariance alignment and boundary seed |
| Offline S10 result | 81.82% accuracy; 82.02% balanced accuracy |

## What to choose

1.  Choose the agreed standard maze number.

2.  Press Enter for subject 10, or type 10.

3.  Use a unique test ID such as S10-EA-01.

4.  Choose the agreed camera view.

5.  Choose the real OpenBCI board and the preflight COM port.

6.  During calibration, alternate sustained left- and right-hand imagery as instructed.

## During and after the run

[ ]  Confirm the console says the checkpoint was trained on subjects [1, 3, 4, 5, 6, 7, 8].

[ ]  At each wall, wait for the prompt before starting imagery.

[ ]  Close the maze normally at the end; wait for EEG and decision files to save.

[ ]  Record observations: __________________________________________________________

Fallback. GUI-only alternative: replace --live-maze with --online in the command above.

SUBJECT 10 UNSEEN MODEL TEST

# Riemann Tangent

Covariance geometry baseline  |  82.04% strict unseen balanced accuracy

## Run the maze

uv run python classifier/run.py --live-maze --decoder riemann --subject 10 --require-unseen --checkpoint models/classical/riemann__7subjects_28runs_22b10fee.joblib

| Item | Value |
| --- | --- |
| Checkpoint | riemann__7subjects_28runs_22b10fee.joblib |
| Training subjects | 1, 3, 4, 5, 6, 7, 8 only |
| Startup calibration | 45 seconds; Riemannian alignment and boundary seed |
| Offline S10 result | 81.82% accuracy; 82.04% balanced accuracy |

## What to choose

1.  Choose the same maze and view used for the other models.

2.  Press Enter for subject 10, or type 10.

3.  Use a unique test ID such as S10-RIEMANN-01.

4.  Choose the real OpenBCI board and the preflight COM port.

5.  Complete the full 45-second alternating imagery calibration.

## During and after the run

[ ]  Verify the unseen guard accepts the checkpoint before calibration.

[ ]  Watch for sustained one-sided predictions; note when they begin.

[ ]  Close the maze normally and wait for the recovered FIF message.

[ ]  Record observations: __________________________________________________________

Fallback. GUI-only alternative: replace --live-maze with --online.

SUBJECT 10 UNSEEN MODEL TEST

# CompactDyn

Compact neural decoder  |  83.33% strict unseen balanced accuracy

## Run the maze

uv run python classifier/run.py --live-maze --decoder deep_compactdyn --subject 10 --require-unseen --checkpoint models/deep/deep_compactdyn__7subjects_28runs_22b10fee.pt

| Item | Value |
| --- | --- |
| Checkpoint | deep_compactdyn__7subjects_28runs_22b10fee.pt |
| Parameters | 31,850 |
| Training subjects | 1, 3, 4, 5, 6, 7, 8 only |
| Startup calibration | 15 seconds; boundary seed only |
| Offline S10 result | 83.33% accuracy and balanced accuracy |

## What to choose

1.  Choose the same standard maze and camera view.

2.  Press Enter for subject 10, or type 10.

3.  Use a unique test ID such as S10-COMPACT-01.

4.  Choose the real OpenBCI board and the preflight COM port.

5.  Complete the 15-second imagery calibration without moving the cap.

## During and after the run

[ ]  Confirm the saved checkpoint loads; it should not start neural training.

[ ]  If training starts, stop with Ctrl+C and check the checkpoint path.

[ ]  Close the maze normally and wait for all diagnostic files to flush.

[ ]  Record observations: __________________________________________________________

Fallback. GUI-only alternative: replace --live-maze with --online.

SUBJECT 10 UNSEEN MODEL TEST

# Sinc Dynamics

Smallest neural decoder  |  81.11% strict unseen balanced accuracy

## Run the maze

uv run python classifier/run.py --live-maze --decoder deep_sinc --subject 10 --require-unseen --checkpoint models/deep/deep_sinc__7subjects_28runs_22b10fee.pt

| Item | Value |
| --- | --- |
| Checkpoint | deep_sinc__7subjects_28runs_22b10fee.pt |
| Parameters | 13,331 |
| Training subjects | 1, 3, 4, 5, 6, 7, 8 only |
| Startup calibration | 15 seconds; boundary seed only |
| Offline S10 result | 81.11% accuracy and balanced accuracy |

## What to choose

1.  Choose the same standard maze and camera view.

2.  Press Enter for subject 10, or type 10.

3.  Use a unique test ID such as S10-SINC-01.

4.  Choose the real OpenBCI board and the preflight COM port.

5.  Complete the 15-second imagery calibration.

## During and after the run

[ ]  Confirm the saved checkpoint loads; it should not start neural training.

[ ]  Keep the participant still because this model has no covariance alignment stage.

[ ]  Close the maze normally and wait for all diagnostic files to flush.

[ ]  Record observations: __________________________________________________________

Fallback. GUI-only alternative: replace --live-maze with --online.

SUBJECT 10 UNSEEN MODEL TEST

# MIRepNet

Pretrained foundation decoder  |  83.16% strict unseen balanced accuracy

## Run the maze

uv run python classifier/run.py --live-maze --decoder mirepnet --subject 10 --require-unseen --checkpoint models/mirepnet/mirepnet__7subjects_28runs_22b10fee.pt

| Item | Value |
| --- | --- |
| Checkpoint | mirepnet__7subjects_28runs_22b10fee.pt |
| Parameters | 5,142,786 |
| Training subjects | 1, 3, 4, 5, 6, 7, 8 only |
| Startup calibration | Choose option 1; 45 seconds; alignment and boundary seed |
| Offline S10 result | 83.61% accuracy; 83.16% balanced accuracy |

## What to choose

1.  Choose the same standard maze and camera view.

2.  Press Enter for subject 10, or type 10.

3.  Use a unique test ID such as S10-MIREPNET-01.

4.  Choose the real OpenBCI board and the preflight COM port.

5.  At MIRepNet live startup, choose 1 for unlabeled calibration.

6.  Complete all 45 seconds of imagery calibration.

## During and after the run

[ ]  Confirm the console says no Subject 10 leakage was detected.

[ ]  Confirm the selected file ends in mirepnet__7subjects_28runs_22b10fee.pt.

[ ]  Do not use mirepnet__finetuned_20260921_100007.pt; it contains S10.

[ ]  Close the maze normally and wait for recovered FIF and decision-log messages.

[ ]  Record observations: __________________________________________________________

AFTER EVERY MODEL

# Save Check Diagnose

Every guarded live launch records console output, raw EEG, predictions, confidence, checkpoint identity, board details, and exceptions. Perform these checks before starting the next model.

## Session completion checklist

[ ]  Confirm the maze report and trace were saved.

[ ]  Confirm the console reports a recovered raw FIF file.

[ ]  Write the diagnostic-session folder name: ______________________________________

[ ]  Write any cap adjustment, interruption, or participant issue in the notes below.

## Diagnose the latest session

uv run python scripts/diagnose_mirepnet_session.py

The command examines the newest folder under recordings/diagnostic-sessions and writes diagnosis.json. Despite the script name, the shared diagnostic recorder is used by every decoder.

## If something fails

[ ]  Do not delete or rename the diagnostic-session folder.

[ ]  Copy the complete error shown in console.log.

[ ]  If COM access fails, close OpenBCI GUI, reconnect the dongle, and rerun preflight.

[ ]  If channels are flat or noisy, fix contact before repeating the model.

[ ]  If --require-unseen stops the launch, do not bypass it; verify the checkpoint path.

[ ]  If the maze closes unexpectedly, run the diagnosis command before restarting.

## Session notes

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________